REPLACE THE FUEL FILLER HOSE ON 73-74 PANTERA

By Frank Hardy 4/4/02 74Pantera@cox.net

5 ½

5/8

2 ¼                         Figure A

5/8

- Raise deck lid. Remove engine cover.

- Remove two screws at rear of fuel tank cover (just in front of driver’s side deck lid strut).

- Lift rear of fuel tank cover. Long thin screwdriver will reach to loosen hose clamp at fuel tank neck.

- Remove driver’s side rear tire.

- Remove gravel shield at front of wheel well.

- Loosen hose clamp on fuel filler neck.

- Slit hose with utility knife and remove. (Carefully. Brittle hose may chunk off and drop into tank.)

- Temporarily plug fuel tank neck with appropriate material.

- Remove four bolts from around fuel filler neck and remove fuel filler neck. Verify diameter of tube. 2 ¼”

- Prepare new hose as indicated in figure A. (2 ¼” ID, 5 ½” long, 5/8” ‘watermelon slice’ trimmed from one half both ends, 180 degrees from each other.)

- Lubricate inside of hose tank-side end and slide hose onto fuel tank neck. Position toward fuel filler neck opening.

- Viewing through fuel filler neck opening, mark hose end for additional trimming for good fit.

- Remove hose. Do final trimming and test fit as in 11.

- Lubricate hose outer end and slide fuel filler neck into outer end. Loosen hose clamps to max and slide over hose.

- Remove temporary plug from fuel tank neck. Slide hose tank-side end onto fuel tank neck. Once started, push firmly in and forward (Fig. C), pushing on fuel filler neck, which will become visible through opening and snap into position.

- Rotate fuel filler neck to align four holes.

- Neck will be askew. Installing upper and lower bolts will bring it into position for rear bolt to fit also. Install front bolt.

- Position and tighten clamps. Reverse 1-5.

# Overhead view

Quarter panel            Tank     Figure B                                Figure C

Filler neck

misfit

Push firmly in and forward